SELECTING CORRECT THREADS, NEEDLES AND TENSIONS

For the best results with your sewing always use:

- Good quality needles – e.g. Smetz and Organ, sharps for woven fabrics and ballpoint for knits, universal are useful but don’t always stitch so well. Change the needle if it’s blunt.

- Good quality threads for seams and finishing – Swan, Coats and Drima are fine and shouldn’t cost more than £1/500m. Guttermein is high quality and strong so more expensive. Use up cheap ones for ZZ.

- Stitch/tension tests with the same fabric and number of layers as your garment. The list below is just a guide and fabrics vary.

- Compatible top/upper and bobbin threads.

| Fabric |  | Thread | Needle | Presser foot pressure | Stitch Length |  |  | Upper Thread Tension |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Only available on some machines | Straight Stitch |  | ZZ L-length W-width |  |
| Light | Nylon | Polyester | 70 (10) sharp | 1 | 2.5/3 |  | L-2/3 W-4/5 | 2/3 |
|  | Cotton | PolyCotton |  |  |  |  |  |  |
|  | Silk | Silk or cotton |  |  |  |  |  |  |
|  | Wool | Polycotton | 80 (12) sharp |  |  |  |  |  |
|  | Knits | Polyester | 70 (10) ballpoint |  |  |  |  |  |
| Medium | Poplin/Calico | Cotton | 80 (12) or 90 (14) sharp | 2 | 3/4 | L-2.5/3.5 W-4/5 |  | 4 |
|  | Gaberdine | Polycotton |  |  |  |  |  |  |
|  | Light Jersey | Polyester | 80 (12) or 90 (14) ballpoint |  |  |  |  |  |
| Heavy | Denim | Cotton | 90 (14) – 100 (16) | 3 | 4 | L-4 W-4/5 |  | 6 |
|  | Tweed | Polyester | 80 (12) – 90 (14) |  |  |  |  |  |
|  | Jersey/Fleece | Cotton |  |  |  |  |  |  |