COMMUNITY SEWING CLESUHN2013

GROWN-ON     ALL-IN-ONE FRONT FACING

This type of facing is constructed by extending the fronts of the garment to include the facing.  The button stand/overlap is also included in the construction. As usual, the facing would have to be interfaced.

Construction for pattern:

- Leaving about 25cm free from edge of paper, trace the Front Block

- Add 3cm for button stand/overlap at Centre Front. Label button stand’ Label X at the top and Y at the bottom.

- From the shoulder point at the neck, point A, measure 7cm. Label this B

- At the waistline from the Centre Front, measure 6cm inwards. Label this C

- Draw a curve line from B to C.

- Fold on the X-Y line, folding the extra paper under the block

- Using your Tracing Wheel, Trace from X-A-B-C-Y

- Open out the folder paper and draw in the facing from the Tracing Wheel impression.

- Trace out the facing for the interfacing, this will include the button stand